# Chapter -4 Memory

Concept

  - Computer’s Memory is complicated System.

  - The memory refers to the physical devices used to store programs (sequence of instructions) or data on a temporary or permanent basis for use in a computer or digital electronic devices.

  - It is made up of large number of cells where each cell stores a bit (binary no 0 or 1).

  - The computer memory is used from starting the PC to turning off the PC.

How it works?

  - Starting a PC.

  - The computer loads data from ROM and performs POST.

  - Then Computer loads BIOS from ROM which checks all the devices connected to CPU (Plug & Play).

  - Then computer loads the Operating system from the HDD in to RAM.

  - When any application we open from HDD, it is loaded on the RAM.

  - If any files are used from that application they also loaded on the RAM

  - After saving the work when file closes, it moves from RAM.

Storage Evaluation Criteria

  - Capacity:

    - Total amount of data that can be stored / loaded on the storage unit.

    - Capacity of Primary storage device is less than the secondary storage device.

  - Access Time:

    - It is a time needed to locate and retrieve the stored data from the storage unit in response to any instruction.

    - Access time of Primary storage device is faster than the secondary storage device.

  - Cost per bit of storage:

    - It is the cost of the storage unit for a given storage capacity.

    - Cost per bit of Primary storage device is higher than the secondary storage device.

  - Volatile:

    - If a storage unit can hold the data even if the PC turns off or power interrupts, it is known as non-volatile memory.

    - If a storage unit loses the data when the PC turns off or power interrupts, it is known as volatile memory.

  - Random Access:

    - If the time to access the data from the storage unit is independent of the location of the storage unit, it is called the Random Access or RAM.

    - Locations of the RAM are easy to access as well as it takes the same amount of time

    - Primary storage units are having the random access at most whereas the secondary storage unit uses the pseudo –random access.

Types of Memory

  - According to the memory storing way it is basically divided in to two parts:

    - Primary / Main/ Simple / Volatile Memory

    - Secondary / Auxiliary / Non-Volatile Memory

Primary Memory Vs Secondary Memory

| Primary Memory | Secondary Memory |
| --- | --- |
| - Stores the data which are currently needed by the CPU | - Information which is not currently processing are resides in secondary memory |
| - Uses semi conductor memory | - Uses magnetic or optical memory |
| - Faster than secondary | - Slower than primary |
| - Higher cost per bit | - Lower cost per bit |
| - Communication with CPU is direct | - Communication with CPU is not direct |
| - Usually contained on the motherboard | - Usually it is an external memory |
| - RAM, ROM etc. | - HDD, CDs. DVDs etc. |

Main / Primary Memory Organization

  - Made up of small storage areas known as locations of cells.

  - Each cell stores the fixed number of bits call word length of memory.

  - In the figure shown besides the no. written on the left of the cell is known as address of the word.

  - Each of the word is of same bit which is known as word length.

  - Each location holds an instruction or data.

  - The addressing starts from 0 and ends at n-1

  - Memory of 1024 bytes starts at 0th location where ends at 1023rd location.

  - The computer OS which are referred as 8bit, 16 bit, 32 bit are basically having that much word length in memory identified as separate address.

  - An amount of data stored on some particular address may get changed or removed but the address of the memory never changes.

  - Real time Example: A cupboard with multiple drawers where each drawers are numbered with a specific drawer no label.

Fixed Word Length Memory

  - In certain computers the memory is designed according to the word length in bytes, so the address is given to each word or a byte. This approach is known as word addressable or fixed word length memory.

  - E.g. if the word length is 4 then the address is given to the 4 bytes. In this situation if we store word “Hi!” which will occupies 3 bytes so will be store on a single address where as if we store the word “Hello !”, which is of 7 characters so it will be stored on two consecutive address bytes in the memory.

  - It is basically used in the large scientific computer for gaining the speed of access.

Merit / Advantage:

    - In this mechanism if the word we want to store an 8 byte long word in the 8 word length memory it uses only one step to do so it is speedy.

    - Provides speedy access.

Disadvantage:

    - In the 8 word length memory if we are having most words of 4-5 bytes, it wastes the memory space.

Variable Word Length Memory

  - In certain computers the memory is designed for storing a single byte for single character or byte, which is known as character addressable or variable word length memory approach.

  - E.g. A word “Surat” will be stored in 5 consecutive memory address blocks where as word “Ahemdabad” will be stored in 9 consecutive address blocks of the memory.

  - It is basically used in the personal use computer to save the memory space.

Merit / Advantage:

  - It optimizes the memory according to the data it stores.

Disadvantage:

  - If we would like to store “Computer” which is of 8 byte or character it require 8 steps to complete the process, so it is slow.

Types of Main / Primary Memory

Random Access Memory (RAM)

  - Basically primary memory of computer is known as RAM

  - It is made up of small ICs on it.

  - The motherboards are normally made in such a way that we can increase the memory if we want just by adding more capacity RAM.

  - It is a volatile memory.

  - When the power turned off the data on RAM are erased.

  - Its cost per bit storage is higher than secondary memory.

  - Available in small sizes : 512mb, 1gb, 2gb. 4gb etc.

  - It is available in two techniques: SRAM and DRAM

Static Random Access Memory (SRAM)

  - Easy to understand

  - Similar to flip flop

  - Advantage: It is high speed

  - Disadvantage: Power Consumption and Heat

Dynamic Random Access Memory (DRAM)

  - Complex mechanism

  - Alternative for SRAM

  - Advantage: Uses Less power

Read Only Memory (ROM)

  - 1 kind of RAM which is non-volatile.

  - It is a chip which stores the data permanently  and cannot be changed easily by user programs.

  - The data are stored by using the fuse-links, which burnt the data permanently on the chip; since it is known as burnt-in data.

  - User can’t change the data.

  - It is also known as field stores, permanent stores or dead stores.

  - The data stored on it; are used frequently.

  - In complex structures the micro program are used to read low-level machine functions and reading the data on ROM.

  - System boot program is such a micro program.

  - The user programmed ROMs are basically divided in two categories as mentioned below:

    - Non-Erasable

        - PROM

    - Erasable

      - EPROM

      - EEPROM

      - UVEPROM / FLASH

Programmable Read Only Memory (PROM)

  - These type of ROMs are categorized in Manufacturer Programmed ROM and User Programmed ROM.

  - The manufacturer PROM is usually burnt by the Manufacturing company which comes with certain electronics equipments.

  - For eg.: System Boot Program, Printer controller software in printers.

  - Manufactured PROMs are basically used when the demand is high.

  - Manufactured PROMs can’t modify by the user.

  - In the user PROM, the user can write or store the data on it.

  - User can write such micro program for performing specific tasks, which are commonly known as PROM.

  - After creation the user PROM can be used as many as time the user wants.

  - The user PROM is basically generated by a special type of device which is known as a PROM Programmer.

  - After creation it is generally treated as ROM only.

  - Once the PROM is designed; it can’t be altered or never losses the data even if power fails.

  - It is non-Volatile Memory,

Erasable Programmable Read Only Memory (EPROM)

  - Disadvantage of ROM / PROM is the data can’t altered.

  - The EPROM overcomes this problem.

  - As the name indicates; the data can be erased.

  - At the time of use the user can only read the data.

  - It is basically used for R&D personnel who frequently changes the micro program for testing the efficiency with new program.

  - In most cases it is treated like ROM where the changes are never to be happen, or happens rarely.

  - It is categorized is two type: EEPROM and UVEPROM.

  - In EEPROM(Electrically EPROM); the stored data are erased by using the high voltage electric pulses.

  - In UVEPROM(Ultra-Violet EPROM); the stored data are erased by using the ultra violet light.

  - The EEPROM is easy to use in comparison of UVEPROM.

  - EEPROM is also known as Flash memory as it is easy to alter the data on it.(USB devices, Pen Drives)

Addressing Modes

What is Addressing Modes?

Addressing modes are various techniques to specify the data for the instruction to be carried by the memory of computer. They are used in various computer architectures. The implementation of these addressing modes is done at the lower level; and it is managed by the operating system.

Types of operands used in Instruction

Immediate Constant (constant integer of 8,16 or 32 bits ,constant values stored in the instruction itself) Register (name of register, register number is encoded with instruction)

Memory (reference to location in memory, memory address is encoded with in instruction, register holds the address of memory location)

Types of addressing modes

- Direct Addressing Mode

- Address field contains address of operand in the instruction

- Effective Address (EA) = Address field (A)

- e.g.  ADD A

  - Add contents of cell A to accumulator

  - Look in memory at address A for operand

- Single memory reference to access data

- No additional calculations to work out effective address

- Limited address space

- Register Addressing Mode

- Operand is held in register named in address filed

- EA = R

- e.g. MOV A, B

  - Move the content of Register B to Register A

- Limited number of registers

- Very small address field needed

  - Shorter instructions

  - Faster instruction fetch

- No memory access

- Very fast execution

- Very limited address space

- Multiple registers helps performance

  - Requires good assembly programming or compiler writing

  - N.B. C programming

    - register int a;

- c.f. Direct addressing

- Register Indirect Addressing Mode

- Memory cell pointed to by address field contains the address of (pointer to) the operand which is a register

- EA = (R)

- e.g. LD Acc, B

- Load the value stored in the memory location of register B to the accumulator

- Operand is in memory cell pointed to by contents of register R

- Large address space (2n)

- One fewer memory access than indirect addressing

- Immediate Addressing Mode

- Operand is part of instruction

- Operand = address field

- e.g. ADD 5

  - Add 5 to contents of accumulator

  - 5 is operand

- No memory reference to fetch data

- Fast

- Limited range

- Implicit Addressing Mode

Instruction

- No need of any operand address

Opcode

Instruction contains only the opcode

- Source and Destination both are fixed as accumulator

- e.g. ADD

  - Add the first two items located on the top of accumulator register and replace them from the accumulator

Concept of Cache Memory and Virtual Memory

Cache Memory

Cache memory is computer memory with very short access time used for storage of frequently used instructions or the data from the storage devices of computer like HDD.

When an application starts or data read or write starts first of all it is being searched with the storage device of computer like HDD or CDROM (magnetic or optical drives). Then this operation shift from the storage device to the faster device RAMS which normally is Dynamic RAM. Then they are served with the cache memory which is actually a RAM of static type (SRAM) more costly and faster than the RAM. This cache memory is used for transmission of such frequently required data to the CPU instead of using RAM or storage devices each time.

This cache memory is further divided in three segment or 3 levels.

The system has cache of three levels (level means that overall cache memory is split into different hardware segments which vary in their processing speed and memory). From RAM data is transferred into cache of 3rd level (L3 cache). L3 cache is a segment of overall cache memory. L3 cache is faster than RAM but slower then L2 cache. To further fasten up the process cache of second order L2 cache are used. They are located at immediate place of processor. But in some of the modern processors L2 cache is inbuilt making the process faster. It should be noted that it is not necessary that a system has 3 levels of cache; it might have 1 or 2 level of cache. At the core level is cache of first level that is L1 cache memory. The commonly used commands/instructions/data is stored in this section of memory. This is built in the processor itself. Thus this is fastest of all the cache memory.

Virtual Memory

In the earlier days the memory of computers was too small so the programmers were working on some large programs with size restrictions. To overcome the above limitation the concept of virtual memory was implemented in 1961 by the group of researchers at Manchester, England. It is implemented by the computer and its operating system, which allows the programmers to use large range of memory or storage address for stored data. The computing system maps the programmer’s virtual address to real hardware storage address. So the programmer is free from thinking about the size of memory.

In addition to managing the mapping of the virtual address to the original address, the computer implements the virtual memory which manages the swapping between the RAM data (active storage) and the HDD data(passive storage). The amount of data in units is read in fixed size of bytes or no. of byte are known as pages. And this process is known as paging. It reduces the overall access of the physical devices and increases the speed and performance of the system.

Short Questions

- How does Computer memory work?

- What is access time?

- What are the basic types of memory?

- Differentiate between primary and secondary memory.

- What is fixed length memory? Explain.

- What is variable length memory? Explain.

- Give full form of: SRAM, DRAM, DDR-DRAM, FCRAM, FPM-DRAM, PROM,EPROM, EEPROM, UVEPROM

- What is Virtual Memory? Explain in short.

Long Questions

- Explain primary or main memory organization in detail.

- Explain RAM in detail

- Explain ROM in detail.

- Explain various addressing modes of computer memory in detail.

- What is Cache Memory? Write a short note on that.